[image: image1.jpg]www.clarkemfg.com
503-829-2156




Clarke Manufacturing Inc. 13388 S. Molalla Forest Rd. Molalla OR. 97038
Part # 1393 ATC350X ALL YEARS 
STOCK CAPACITY FUEL TANK

[image: image2.jpg]A0 TANK INSTALLATION 0

Install fuel tank ONLY in a well ventilated area. Clean inside of tank
thoroughly before installation. DO NOT start or operate the vehicle

if there are any fuel leaks. We recommend that installation be
performed by a qualified mechanic. If you have any questions
CALL FIRST, before attempting to install.





Thank you for purchasing a Clarke fuel tank.  This tank is intended use on the ATC350X.  There may be some plastic shavings left in the tank from the manufacturing process.  You can use compressed air to blow out the tank.  You may also want to rinse out the tank using a small amount of fuel before installing it.
                                         [image: image3.jpg]


                                       
PLEASE SEE THE REVERSE SIDE OF THIS PAGE FOR INSTALLATION INSTRUCTIONS
1.) Shut off fuel supply and disconnect fuel line and remove the original tank and petcock.  Drain your old tank in a well ventilated area. 
2.) Install your original petcock to the new tank carefully threading it onto the supplied aluminum petcock adapter.  

3.) Slide the tank onto the front rubber mounting pucks.  If the tunnel is a tight fit, turn your petcock to off
and a small amount of air can be put into the tank using compressed or aspirated air.  Put the air in through the gas cap vent.  This will spread the tank out slightly and allow it to slip on.

      4.) Now hook up the fuel line, make sure that the petcock is turned to the to the OFF position and fill the

 tank with fuel.
      5.) Check for leaks.
      6.) Now install the factory air scoops using the supplied screws.

      7.) Re-install the rear fender and seat assembly. Make sure to “plug in” the side panels to
      the rear tabs of the tank.
      8.) You are now ready for riding. Enjoy your new Clarke tank.







