[image: image1.jpg]www.clarkemfg.com
503-829-2156




Clarke Manufacturing Inc. 13388 S. Molalla Forest Rd. Molalla OR. 97038

Part # 11600 CRF450R FUEL INJECTED 2009-2010

AND CRF250R 2010

2.3 GALLON FUEL TANK

[image: image2.jpg]A0 TANK INSTALLATION 0

Install fuel tank ONLY in a well ventilated area. Clean inside of tank
thoroughly before installation. DO NOT start or operate the vehicle

if there are any fuel leaks. We recommend that installation be
performed by a qualified mechanic. If you have any questions
CALL FIRST, before attempting to install.





Thank you for purchasing a Clarke fuel tank.  The following instructions will help you install this tank on a CRF450R 2009-2010 OR CRF250R 2010 four stroke motorcycle.  There may be some plastic shavings left in the tank from the manufacturing process.  You can use compressed air to blow out the tank.  You may also want to rinse out the tank using a small amount of fuel before installing it. This tank will not use your original front tank mount but instead utilizes a pair of “L” shaped brackets to hold the tank in the frame and prevent it from lifting. The instructions on the back side of this page will help you to quickly install your new tank 

[image: image3.jpg]


                                    [image: image4.jpg]



         PLEASE SEE THE REVERSE SIDE OF THIS PAGE FOR INSTALLITON INSTRUCTIONS

IMPORTANT: THE MAXIMUM TORQUE SPECS FOR THE FUEL PUMP BOLTS IS 5 FT LBS. 

DO NOT EXCEED 5 FT. LBS OF TORQUE WHEN RE-INSTALLING THE FUEL PUMP OR YOU WILL SPIN THE BRASS INSERTS AND VOID THE WARRANTY ON THIS TANK.
1.) Begin by removing your old tank, shrouds, seat and fuel injection unit.  Drain your old tank in a well ventilated area.  You may want to refer to your owners/service manual for proper removal and installation of the fuel pump unit.

2.) Remove the fuel pump assembly from your original tank.   A new o-ring is supplied with this tank to use in place of the old unit.  It goes in the outer most ring around the fuel pump hole.  Test fit it first and stretch it a bit if needed to seat properly. Now, using the stock bolts, mount the injection unit to the new Clarke tank making sure the seal is not bound and fits tight.  A little all purpose oil will aid installation.  After you make sure everything is aligned and secure with no o-ring binding, tighten down all the bolts for the unit.  

3.)  Now put a small amount of fuel into the tank and tip it forward so the gas is located over the fuel pump area.  Check for any leakage around the pump seal.  

4.) Transfer the rear seat mount stud to the new tank.  The tank is now ready for installation.

5.) There are 2 rubber tank pads on the existing frame rails.  These will work with this tank but need to be   slid forward on the rails to help the tank fit better.  

6.) Remove the “L” shaped brackets from the Clarke fuel tank.

7.) Slip the tank carefully into the frame and then check to make sure you have no pinched fuel line

      or wiring interference.  

8.) Re-install the “L” brackets to the tank and tighten the bolts to assure that the fuel tank is secure.

9.) Re-Attach the rubber tank strap to its original location.

     10.) Install the radiator shrouds.

     11.) Now fill the tank with fuel and check for any leaks.

     12.)  Re-install the seat.

     13.)  Go back and make sure all bolts and screws are secure.  You are now ready for riding.  

     Enjoy your new Clarke tank.  

