[image: image1.jpg]www.clarkemfg.com
503-829-2156




Clarke Manufacturing Inc. 13388 S. Molalla Forest Rd. Molalla OR. 97038
Part # 1339 1996-2001 YZ250 and Y125
3.1 GALLON FUEL TANK

[image: image2.jpg]A0 TANK INSTALLATION 0

Install fuel tank ONLY in a well ventilated area. Clean inside of tank
thoroughly before installation. DO NOT start or operate the vehicle

if there are any fuel leaks. We recommend that installation be
performed by a qualified mechanic. If you have any questions
CALL FIRST, before attempting to install.





Thank you for purchasing a Clarke fuel tank.  The following instructions will help you install this tank on a 1996-2001 YZ250 or YZ125 motorcycle.  There may be some plastic shavings left in the tank from the manufacturing process.  You can use compressed air to blow out the tank.  You may also want to rinse out the tank using a small amount of fuel before installing it. The instructions on the back side of this page will help you to install your new tank properly.
                                    
                                      [image: image3.jpg]



PLEASE SEE THE REVERSE SIDE OF THIS PAGE FOR INSTALLATION INSTRUCTIONS

1.) Begin by removing your old tank, shrouds and petcock.  Drain your old tank in a well ventilated area.  

2.) Install your original petcock in the new tank using the new petcock gasket in combination with the original “oval o-ring” that is currently on your OE petcock. 
3.) Transfer the original set of front mounts from your original tank to the new tank using the hardware that is supplied by the original manufacturer.
4.)  Now place the tank over the frame and push it into position. Align the bushing area with the mounting holes on the frame.  These can be difficult to view so be patient and work it until you can get one bolt partially started, then move to the other side and start the other bolt in.  Once both mounting bolts are started go back and tighten them down.
5.) Transfer the rear seat mount stud and the rear strap mount screw from your original tank

 to the new tank.

      6.) Install all shroud bolts loosely on the tank section using the original shroud bolts and loosely install
      the radiator mount bolts.  
      7.) Now go back and tighten the shroud and radiator mount bolts.  Attach the rubber strap to the rear screw.

      8.) Turn the petcock to the OFF position and fill the tank with fuel.

      9.) Check for leaks.

      10.) Re-install the seat and fasten the seat mount bolts securely.
      11.) You are now ready for riding. Enjoy your new Clarke tank.













